AUTONOMOUS TURRET PROJECT

ESP32 Setup & Development Documentation  ·  v2

dronedome  ·  192.168.68.78  ·  Ubuntu 24.04 LTS

Gautham Chandra  ·  github.com/Gautham004/Turrent

# SECTION 1 — QUICK START: Upload Code to ESP32

Everything you need to go from powered-off ESP32 to running new code. Two methods: VS Code button (easiest) or terminal command.

## 1.1  Network & Device Reference

| Parameter | Value |
| --- | --- |
| dronedome IP | 192.168.68.78 |
| ESP32 IP | 192.168.68.97 |
| ESP32 MAC | A0:B7:65:DC:60:A4 |
| USB Port | /dev/ttyUSB0 |
| Baud Rate | 115200 |
| Project Path | ~/turret/esp32 |
| Source File | ~/turret/esp32/src/main.cpp |

## 1.2  Method 1 — VS Code Upload Button (Easiest)

- Open VS Code with Remote SSH connected to dronedome

- File → Open Folder → /home/gautham/turret (open the full turret folder)

- Click the PlatformIO icon in the left sidebar (alien/ant icon)

- Under PROJECT TASKS → esp32dev → click Upload

- Or use the bottom status bar: click the → arrow icon

- Watch the terminal — upload completes in ~10 seconds

ℹ VS Code Status Bar Icons

▸ ✓ (checkmark) → Build only

▸ → (arrow) → Build and Upload

▸ 🔌 (plug) → Serial Monitor

▸ These appear in the blue bar at the very bottom of VS Code

## 1.3  Method 2 — Terminal Upload

If the VS Code button isn't available, use the terminal:

cd ~/turret/esp32

sudo chmod 666 /dev/ttyUSB0 && pio run --target upload --upload-port /dev/ttyUSB0

Open serial monitor after upload:

sudo chmod 666 /dev/ttyUSB0 && pio device monitor --port /dev/ttyUSB0 --baud 115200

Press Ctrl+C to exit the monitor. Always close the monitor before uploading.

## 1.4  Editing Code

- Open ~/turret/esp32/src/main.cpp in VS Code

- Edit the code

- Save (Ctrl+S)

- Click → upload button or run the terminal command above

⚠ Only one .cpp file in src/ at a time

▸ PlatformIO compiles ALL .cpp files in the src/ folder

▸ Having main.cpp AND test.cpp causes 'multiple definition of setup()' error

▸ Delete or move unused files before uploading

▸ To delete a test file: rm ~/turret/esp32/src/test.cpp

## 1.5  platformio.ini (Current Config)

[env:esp32dev]

platform = espressif32

board = esp32dev

framework = arduino

upload_protocol = esptool

monitor_speed = 115200

# SECTION 2 — FULL SETUP WALKTHROUGH

Complete documentation of everything set up in this session — hardware inventory, power system, PlatformIO install, and firmware development sequence.

## 2.1  Hardware Inventory

| Component | Status | Turret Role |
| --- | --- | --- |
| Arduino Uno (clone) | ✅ | Dev/testing board (CH340 USB chip) |
| Arduino Nano (clone) | ✅ | Final embedded servo controller |
| Arduino Ethernet | ✅ | Spare — potential v2 WiFi control |
| L293D Motor Shield | ✅ | Flywheel DC motor control |
| ESP32 x2 (WROOM-32) | ✅ | IFF BLE scanner + beacon |
| BTF-100-5 PSU (5V 20A) | ✅ | Primary indoor power supply |
| Mighty Max ML7-12 SLA | ✅ | Portable/outdoor power backup |
| AA Battery Holders x8 | ✅ | Logic power (prototype use) |
| Breadboard PSU Module | ✅ | Power injection into breadboard |
| Bergen 9ft Power Cord | ✅ | Mains AC feed to BTF PSU |
| 5V Relay Module | ✅ | Nerf flywheel enable switch |
| KY-037 Sound Sensor | ✅ | Wake trigger (sound-activated) |
| PIR Sensors x3 | ✅ | Wake trigger (motion-activated) |
| MPU-6050 IMU | ✅ | Gimbal angle feedback |
| 4x4 Matrix Keypad | ✅ | Manual mode switching / arming |
| 16x2 LCD Display | ✅ | Status display |
| Joystick Module | ✅ | Manual override pan/tilt control |
| Rotary Encoder | ✅ | Mode selection knob |
| 37-in-1 Sensor Kit | ✅ | Misc sensors (buzzer, IR, hall effect) |
| 3D Printer + Filament | ✅ | Pan-tilt gimbal mount fabrication |
| Dupont Jumper Wires | ✅ | PSU to breadboard connections |
| Breadboard | ✅ | Prototype wiring |
| MG996R Servos x2 | ⬜ | Pan and tilt axes — still needed |
| USB Webcam | ⬜ | YOLOv8 vision — still needed |
| Nerf Gun | ⬜ | Firing platform — still needed |

## 2.2  Power System

### BTF-100-5 PSU Terminal Wiring

| Wire / Terminal | Connection |
| --- | --- |
| Black wire | L terminal (Hot) |
| White wire | N terminal (Neutral) |
| Green wire | ⏚ terminal (Ground/Earth) |
| +V terminals | 5V DC positive output |
| -V terminals | 5V DC negative / common ground |
| ADJ terminal | Fine voltage trim — leave untouched |

⚠ Critical Safety Steps

▸ Voltage switch MUST be set to 110V (was shipped set to 220V)

▸ Verify 5V output with multimeter before connecting any electronics

▸ All GND terminals must share a common ground across every subsystem

▸ NUC runs on its own Dell 45W adapter — never connect to BTF PSU

▸ Do not power servos through breadboard traces — connect directly to PSU terminals

### Indoor Power Architecture

Wall Outlet (110V AC)

└── BTF-100-5 PSU (5V 20A)

├── Breadboard PSU Module → breadboard + rail

│     ├── Arduino Nano / ESP32

│     ├── LCD, Sensors, Relay

├── Servo power (direct from PSU terminals)

│     ├── Pan Servo (MG996R)

│     └── Tilt Servo (MG996R)

└── Common GND (all subsystems tied together)

ℹ Current Budget

▸ Estimated draw: ~3A (servos + logic + sensors)

▸ PSU capacity: 20A ¦ Headroom: ~17A

▸ SLA battery used only for portable/outdoor deployments

## 2.3  Software Setup on dronedome

### PlatformIO Installation

pip install platformio --break-system-packages

### Project Initialization

mkdir -p ~/turret/esp32/src

cd ~/turret/esp32

pio project init --board esp32dev

### Serial Port — Permanent Permission Fix

sudo usermod -a -G dialout gautham

sudo usermod -a -G tty gautham

Log out and back in after running. Temporary fix: sudo chmod 666 /dev/ttyUSB0

### VS Code PlatformIO Extension

Install the PlatformIO IDE extension in VS Code for one-click uploads:

- Ctrl+Shift+X → search PlatformIO IDE → Install

- File → Open Folder → /home/gautham/turret (always open the root turret folder)

- Click the PlatformIO icon (left sidebar) to access upload/monitor buttons

- Bottom status bar shows: ✓ Build  →  Upload  🔌 Monitor

## 2.4  Firmware Development Sequence

### Stage 1 — Blink Test

First upload confirmed the board was functional. Onboard LED on GPIO2 blinked at 500ms with serial output confirming:

- ESP32-D0WD-V3 revision 3.1

- CH340 USB chip on /dev/ttyUSB0

- Serial communication at 115200 baud working

### Stage 2 — WiFi Connection

| Parameter | Value |
| --- | --- |
| SSID | HomeWifi |
| Assigned IP | 192.168.68.97 |
| MAC Address | A0:B7:65:DC:60:A4 |

### Stage 3 — OTA (Attempted, Reverted)

OTA wireless upload was set up and confirmed working (upload via WiFi to 192.168.68.97). However, OTA was reverted back to wired USB upload for simplicity during active development. OTA can be re-enabled later when the turret is physically mounted.

### Stage 4 — Hello World Test

Confirmed full upload pipeline working end-to-end with a simple 10-iteration print test:

#include <Arduino.h>

void setup() {

Serial.begin(115200);

delay(1000);

for (int i = 1; i <= 10; i++) {

Serial.printf("[%d/10] Hello World!\n", i);

delay(500);

}

Serial.println("--- Done ---");

}

void loop() { }

ℹ Result

▸ Upload: SUCCESS (9.33 seconds)

▸ Serial output: [1/10] through [10/10] Hello World! confirmed

▸ Full pipeline verified: edit → save → upload → monitor

# SECTION 3 — SYSTEM ARCHITECTURE

## 3.1  Overall System

NUC (dronedome — 192.168.68.78)

├── USB → Webcam (YOLOv8 person detection)

├── USB Serial → ESP32 #1 (servo control + BLE IFF scanner)

│     ├── Core 0: BLE scanning (IFF friend detection)

│     └── Core 1: PWM servo control + serial comms

ESP32 #2 (carried by friend)

└── BLE beacon (broadcasts MAC for IFF whitelist)

Turret Hardware

├── Pan Servo (MG996R)

├── Tilt Servo (MG996R)

├── Relay Module → Nerf flywheel

├── 16x2 LCD → status display

└── Joystick → manual override

## 3.2  Weekly Build Roadmap

| Phase | Goal |
| --- | --- |
| Week 1 | 3D simulation — Kalman filter + PID tracking (COMPLETE) |
| Week 2 | Live camera — YOLOv8 person detection on dronedome |
| Week 3 | ESP32 serial bridge — Python to servo PWM |
| Week 4 | BLE IFF system — friend-or-foe identification |
| Week 5 | Full integration — detect → IFF → track → fire |
| Week 6 | Polish — benchmarks, demo video, GitHub README |

# SECTION 4 — QUICK REFERENCE

## 4.1  Common Commands

| Action | Command |
| --- | --- |
| SSH into dronedome | ssh gautham@192.168.68.78 |
| Navigate to project | cd ~/turret/esp32 |
| Upload via USB | sudo chmod 666 /dev/ttyUSB0 && pio run --target upload --upload-port /dev/ttyUSB0 |
| Serial monitor | sudo chmod 666 /dev/ttyUSB0 && pio device monitor --port /dev/ttyUSB0 --baud 115200 |
| Clean build | pio run --target clean |
| Check USB port | ls /dev/ttyUSB* |
| Fix port permissions | sudo chmod 666 /dev/ttyUSB0 |
| Fix black screen RDP | sudo systemctl restart gdm |
| Ping ESP32 | ping 192.168.68.97 |
| Delete extra src file | rm ~/turret/esp32/src/test.cpp |

## 4.2  Troubleshooting

| Problem | Fix |
| --- | --- |
| Permission denied on /dev/ttyUSB0 | sudo chmod 666 /dev/ttyUSB0 |
| Port busy / IO error | sudo fuser -k /dev/ttyUSB0 then retry |
| multiple definition of setup() | Delete extra .cpp files from src/ folder |
| Upload fails — board not responding | Hold BOOT button while upload starts, release at Connecting... |
| RDP black screen | sudo systemctl restart gdm |
| PlatformIO not finding project | File → Open Folder → /home/gautham/turret |
| sdasudo not found | You typed sdasudo — it should be sudo |

## 4.3  Next Steps

- Wire MG996R servos to ESP32 GPIO pins and test PWM control

- Write Python serial bridge on dronedome to send pan/tilt angle commands

- Connect webcam and verify YOLOv8 detects persons

- Integrate Kalman filter output with serial servo commands

- Set up second ESP32 as BLE beacon for IFF system

- Print pan-tilt mount and assemble gimbal

Autonomous Turret Project  ·  Gautham Chandra  ·  github.com/Gautham004/Turrent  ·  v2